Bài 1:

Bài 2:

Bài 3:

Bài 4: (v1)

Bài 4: (v2)

Bài 5:

Bài 6:

Bài 7:

Bài 8:

Bài 9:

Bài 10:

Bài 11:

Bài 12:

Bài 13:

Bài 14:

Bài 15:

Bài 16:

| import cv2 import numpy as np import RPi.GPIO as GPIO import time import os import subprocess # Cấu hình LCD LCD_PINS = {'RS': 23, 'E': 27, 'D4': 18, 'D5': 17, 'D6': 14, 'D7': 3, 'BL': 2} BT_1 = 21 BT_2 = 26 RELAY_1 = 16 # Chân điều khiển Relay 1 RELAY_2 = 12 # Chân điều khiển Relay 2 LCD_WIDTH = 16 LCD_CHR = True LCD_CMD = False LCD_LINE_1 = 0x80 LCD_LINE_2 = 0xC0 E_PULSE = 0.0005 E_DELAY = 0.0005 # Khởi tạo GPIO và LCD GPIO.setmode(GPIO.BCM) GPIO.setwarnings(False) GPIO.setup(BT_1, GPIO.IN, pull_up_down=GPIO.PUD_UP) GPIO.setup(BT_2, GPIO.IN, pull_up_down=GPIO.PUD_UP) GPIO.setup(RELAY_1, GPIO.OUT) GPIO.setup(RELAY_2, GPIO.OUT) GPIO.output(RELAY_1, GPIO.LOW) # Relay OFF (active HIGH - LOW = OFF) GPIO.output(RELAY_2, GPIO.LOW) # Relay OFF def lcd_init(): for pin in LCD_PINS.values(): GPIO.setup(pin, GPIO.OUT) for byte in [0x33, 0x32, 0x28, 0x0C, 0x06, 0x01]: lcd_byte(byte, LCD_CMD) def lcd_string(message, line=LCD_LINE_1): message = message.center(LCD_WIDTH, " ") lcd_byte(line, LCD_CMD) for char in message: lcd_byte(ord(char), LCD_CHR) def lcd_clear(): lcd_byte(0x01, LCD_CMD) def lcd_byte(bits, mode): GPIO.output(LCD_PINS['RS'], mode) for bit_num in range(4): GPIO.output(LCD_PINS[f'D{bit_num+4}'], bits & (1 << (4 + bit_num)) != 0) time.sleep(E_DELAY) GPIO.output(LCD_PINS['E'], True) time.sleep(E_PULSE) GPIO.output(LCD_PINS['E'], False) time.sleep(E_DELAY) for bit_num in range(4): GPIO.output(LCD_PINS[f'D{bit_num+4}'], bits & (1 << bit_num) != 0) time.sleep(E_DELAY) GPIO.output(LCD_PINS['E'], True) time.sleep(E_PULSE) GPIO.output(LCD_PINS['E'], False) time.sleep(E_DELAY) | def lcd_display_string(message, line): lcd_byte(LCD_LINE_1 if line == 1 else LCD_LINE_2, LCD_CMD) for char in message: lcd_byte(ord(char), LCD_CHR) def capture_image(cap): ret, frame = cap.read() if ret: # Tạo tên file với timestamp để tránh trùng lặp timestamp = time.strftime("%Y%m%d-%H%M%S") filename = f"/home/pi/captured_image_{timestamp}.jpg" cv2.imwrite(filename, frame) return frame, filename return None, None def detect_colors(image): # Chuyển ảnh sang HSV hsv = cv2.cvtColor(image, cv2.COLOR_BGR2HSV) # Định nghĩa khoảng màu đỏ trong HSV lower_red1 = np.array([0, 120, 70]) upper_red1 = np.array([10, 255, 255]) lower_red2 = np.array([170, 120, 70]) upper_red2 = np.array([180, 255, 255]) # Định nghĩa khoảng màu xanh lá cây trong HSV lower_green = np.array([35, 50, 50]) upper_green = np.array([85, 255, 255]) # Tạo mask cho màu đỏ mask_red1 = cv2.inRange(hsv, lower_red1, upper_red1) mask_red2 = cv2.inRange(hsv, lower_red2, upper_red2) red_mask = mask_red1 + mask_red2 # Tạo mask cho màu xanh lá cây green_mask = cv2.inRange(hsv, lower_green, upper_green) # Đếm số pixel red_pixels = cv2.countNonZero(red_mask) green_pixels = cv2.countNonZero(green_mask) return red_pixels, green_pixels def control_relays(red_pixels, green_pixels): if red_pixels > green_pixels: GPIO.output(RELAY_1, GPIO.HIGH) # Bật Relay 1 (active HIGH) GPIO.output(RELAY_2, GPIO.LOW) # Tắt Relay 2 return "RED > GREEN", 1 else: GPIO.output(RELAY_1, GPIO.LOW) # Tắt Relay 1 GPIO.output(RELAY_2, GPIO.HIGH) # Bật Relay 2 return "GREEN >= RED", 2 def show_image(filename): try: # Mở ảnh bằng trình xem ảnh mặc định trên Raspberry Pi subprocess.run(["xdg-open", filename]) except Exception as e: print(f"Error opening image: {e}") | def main(): cap = cv2.VideoCapture(0) print("Capture is ok") lcd_init() lcd_clear() lcd_display_string("System Ready", 1) captured_image = None last_image_path = None try: while True: # Kiểm tra nút BT1 (chụp ảnh) if GPIO.input(BT_1) == GPIO.LOW: print("Button 1 pressed - Capturing image...") captured_image, image_path = capture_image(cap) if captured_image is not None: lcd_clear() lcd_display_string("Image captured!", 1) lcd_display_string("Saving...", 2) last_image_path = image_path time.sleep(1) # Hiển thị ảnh vừa chụp lcd_clear() lcd_display_string("Showing image...", 1) show_image(image_path) time.sleep(1) else: lcd_clear() lcd_display_string("Capture failed!", 1) time.sleep(1) # Kiểm tra nút BT2 (đếm pixel và điều khiển relay) if GPIO.input(BT_2) == GPIO.LOW and captured_image is not None: print("Button 2 pressed - Processing image...") red_pixels, green_pixels = detect_colors(captured_image) result, relay_state = control_relays(red_pixels, green_pixels) lcd_clear() lcd_display_string(f"R:{red_pixels} G:{green_pixels}", 1) lcd_display_string(result, 2) print(f"Red pixels: {red_pixels}, Green pixels: {green_pixels}") print(f"Relay {relay_state} activated") time.sleep(0.5) # Chống nhiễu nút nhấn time.sleep(0.1) # Giảm tải CPU except KeyboardInterrupt: print("Program terminated") finally: cap.release() GPIO.output(RELAY_1, GPIO.LOW) # Tắt Relay khi kết thúc GPIO.output(RELAY_2, GPIO.LOW) # Tắt Relay khi kết thúc GPIO.cleanup() lcd_clear() if __name__ == "__main__": main() |
| --- | --- | --- |